%% EXPERIMENT 1: TEM, TE AND TM WAVEGUIDE MODES

clc; clear; close all;

c = 299792458;

a = 22.86e-3; b = 10.16e-3;

f0 = 10e9;

fc10 = c/(2*a);

fc11 = c/2*sqrt((1/a)^2+(1/b)^2);

x = linspace(0,a,100);

y = linspace(0,b,60);

[X,Y] = meshgrid(x,y);

% Normalized analytical field patterns

Ey10 = sin(pi*X/a);

Hx10 = Ey10;

Ez11 = sin(pi*X/a).*sin(pi*Y/b);

Hx11 = (pi/a)*cos(pi*X/a).*sin(pi*Y/b);

Hy11 = (pi/b)*sin(pi*X/a).*cos(pi*Y/b);

Htm11 = sqrt(Hx11.^2+Hy11.^2);

Htm11 = Htm11/max(Htm11(:));

% Frequency sweep

f = linspace(8e9,20e9,250);

beta10 = 2*pi*f/c.*sqrt(max(0,1-(fc10./f).^2));

beta11 = 2*pi*f/c.*sqrt(max(0,1-(fc11./f).^2));

beta11(f<fc11) = NaN;

figure('Color','w','Position',[30 30 1200 700]);

% 1. TE10 electric field

subplot(2,4,1);

imagesc(x*1e3,y*1e3,Ey10);

set(gca,'YDir','normal'); colorbar;

xlabel('x (mm)'); ylabel('y (mm)');

title('TE_{10}: Electric Field');

% 2. TE10 magnetic field (normalized Hx magnitude)

subplot(2,4,2);

imagesc(x*1e3,y*1e3,abs(Hx10));

set(gca,'YDir','normal'); colorbar;

xlabel('x (mm)'); ylabel('y (mm)');

title('TE_{10}: Magnetic Field');

% 3. TM11 electric field

subplot(2,4,3);

imagesc(x*1e3,y*1e3,Ez11);

set(gca,'YDir','normal'); colorbar;

xlabel('x (mm)'); ylabel('y (mm)');

title('TM_{11}: Electric Field');

% 4. TM11 transverse magnetic field

subplot(2,4,4);

imagesc(x*1e3,y*1e3,Htm11);

set(gca,'YDir','normal'); colorbar;

xlabel('x (mm)'); ylabel('y (mm)');

title('TM_{11}: Magnetic Field');

% 5. Propagation constant

subplot(2,4,5);

plot(f/1e9,beta10,'b','LineWidth',1.5); hold on;

plot(f/1e9,beta11,'r','LineWidth',1.5);

xline(fc10/1e9,'b--'); xline(fc11/1e9,'r--');

xlabel('Frequency (GHz)'); ylabel('\beta (rad/m)');

title('Propagation Constant');

legend('TE_{10}','TM_{11}','Location','northwest');

grid on;

% 6. Cutoff frequencies

subplot(2,4,6);

bar([fc10 fc11]/1e9);

set(gca,'XTickLabel',{'TE_{10}','TM_{11}'});

ylabel('Frequency (GHz)'); title('Cutoff Frequencies');

grid on;

% 7. Mode status at 10 GHz

subplot(2,4,7);

status = [f0>fc10, f0>fc11];

bar(double(status));

set(gca,'XTickLabel',{'TE_{10}','TM_{11}'},...

'YTick',[0 1],'YTickLabel',{'Below cutoff','Propagates'});

ylim([0 1.2]); title('Mode Status at 10 GHz');

grid on;

% 8. Operating frequency relative to cutoff

subplot(2,4,8);

bar([f0-fc10 f0-fc11]/1e9);

set(gca,'XTickLabel',{'TE_{10}','TM_{11}'});

ylabel('f_0-f_c (GHz)');

title('Frequency Relative to Cutoff');

yline(0,'k-'); grid on;

sgtitle('Experiment 1: Rectangular Waveguide Mode Analysis');

fprintf('TE10 cutoff frequency = %.3f GHz\n',fc10/1e9);

fprintf('TM11 cutoff frequency = %.3f GHz\n',fc11/1e9);

fprintf('Operating frequency = %.3f GHz\n',f0/1e9);

fprintf('TE10: %s\n',string(status(1)*"Propagates" + ...

~status(1)*"Below cutoff"));

fprintf('TEM mode is not supported by a hollow rectangular waveguide.\n');